Part C — Few-shot Design

Model: Claude Opus 4.1

Temperature: 0.2

Max Tokens: 600

1. Compare:

• zero-shot

Prompt A:

Design a temperature data-acquisition system with these specs:

- 10 Hz sampling rate

- 12-bit ADC, 3.3V supply

- Detect abnormal temperature readings

- Include: architecture, components, fault detection, risks

Output:

### Temperature Data-Acquisition System Design

Architecture Overview:

- Microcontroller: STM32L476 (low-power ARM Cortex-M4)

- Sensor: PT1000 RTD (Pt100 alternative) or NTC thermistor

- ADC: 12-bit internal ADC on STM32

- Interface: SPI/UART for data logging

Key Components:

- Microcontroller: STM32L476 with integrated 12-bit ADC, 3.3V operation

- Temperature Sensor: PT1000 RTD with 100Ω at 0°C (stable, accurate)

- Analog Front-End: Simple RC filter for noise rejection (cutoff ~50Hz for 10Hz sampling)

- Power Supply: 3.3V linear regulator with 10µF + 100nF filtering

- Communication: UART or SPI interface for data transmission

Fault Detection Strategy:

- Open Circuit Detection: Monitor ADC voltage limits—if reading exceeds predefined threshold, flag as open

- Short Circuit Detection: If ADC reads below minimum expected voltage, flag as short

- Range Checking: Implement digital filtering (median filter over 5 samples) to detect erratic readings

- Timeout Detection: If no new reading received within 200ms, flag sensor timeout

Abnormal Measurement Detection:

- Implement running mean and standard deviation calculation

- Flag readings >2σ from mean as abnormal

- Use hysteresis to prevent chatter (±3% band)

- Log abnormal events with timestamp

Main Risks & Mitigation:

- Sensor Drift: Periodic calibration routine quarterly

- ADC Quantization Noise: Averaging multiple readings

- EMI/Ground Issues: Separate analog ground plane, shield sensor lines

- Temperature Drift of Components: Use temperature-compensated reference

- Power Supply Ripple: Multi-stage filtering, linear regulator

• one-shot

Prompt B:

EXAMPLE - Humidity Sensor System:

Microcontroller: STM32L0 → DHT22 sensor → UART logging

Fault detection: CRC checksum, timeout detection (>500ms)

Risks: Sensor failures, I2C noise

Now design a temperature data-acquisition system with these specs:

- 10 Hz sampling rate

- 12-bit ADC, 3.3V supply

- Detect abnormal temperature readings

- Include: architecture, components, fault detection, risks

Output:

### Temperature Data-Acquisition System Design

Context from Example:
 Similar to the humidity system provided, but adapted for analog temperature sensing with stricter fault detection.

Architecture Overview:

- Microcontroller: STM32L476 (same family as example, proven reliability)

- Sensor: NTC Thermistor (10kΩ at 25°C) with voltage divider

- ADC: 12-bit, 3.3V reference

- Output: UART logging to SD card or cloud gateway

Key Components:

- Microcontroller: STM32L476 with built-in 12-bit ADC, 3.3V supply

- Temperature Sensor: NTC thermistor with linearization lookup table

- Voltage Divider: 10kΩ pullup resistor + thermistor to ground

- Anti-Aliasing Filter: RC low-pass (R=1kΩ, C=100nF) for ~1.6kHz cutoff

- UART Interface: Serial output at 115200 baud

Fault Detection Strategy:
 Following the CRC/timeout approach from the humidity example:

- CRC-16 Checksum: Each ADC reading paired with computed CRC

- Timeout Detection: Flag if no reading received for >150ms (1.5× expected interval)

- Saturation Detection: Monitor ADC for max/min values indicating sensor failure

- Thermistor Open/Short: Check voltage divider output—open circuit → ADC reads VCC, short → ADC reads 0V

Abnormal Measurement Detection:

- Statistical method: Calculate rolling mean and variance

- Flag if reading deviates >2.5σ from mean

- Additional sanity check: Temperature within -10°C to +50°C range

- Log abnormal readings with CRC and timestamp

Main Risks & Mitigation:

- Thermistor Nonlinearity: Mitigate with Steinhart-Hart coefficients in lookup table

- Sensor Aging: Use reference calibration points at known temperatures (ice, boiling water)

- Voltage Divider Drift: Select 1% tolerance resistors, temperature-compensated design

- I2C Bus Failures (if applicable): Implement bus watchdog, NACK detection

- Power Supply Noise: Multi-stage filtering, ceramic + electrolytic capacitors

• three-shot

Prompt C:

EXAMPLE 1 - Humidity Sensor:

Microcontroller: ATmega328P → DHT22 → UART logging

Fault detection: CRC checksum, timeout detection

EXAMPLE 2 - Pressure Sensor (Analog):

Microcontroller: STM32L476 → 4-20mA transducer → 12-bit ADC

Fault detection: Saturation detection, open-circuit check

EXAMPLE 3 - Temperature Logger (Thermistor):

Microcontroller: nRF52840 → NTC thermistor → ADC

Fault detection: Short/open circuit check, lookup table for nonlinearity

Now design a temperature data-acquisition system with these specs:

- 10 Hz sampling rate

- 12-bit ADC, 3.3V supply

- Detect abnormal temperature readings

- Include: architecture, components, fault detection, risks

Output:

### Temperature Data-Acquisition System Design

Learning from Examples:

- Example 1 (Humidity): Digital interface, CRC checking, timeout detection

- Example 2 (Pressure): Analog signal chain, ADC usage, saturation detection

- Example 3 (Temperature/Thermistor): NTC-specific circuit, nonlinearity handling

Optimal Architecture:

- Microcontroller: nRF52840 (lowest power, excellent for 10Hz sampling)

- Sensor: NTC Thermistor 10kΩ (passive, low-cost, proven)

- ADC: 14-bit internal (nRF52), oversampled to 12-bit

- Power: 3.3V coin cell or regulated supply

Key Components:

- MCU: nRF52840 with integrated 12/14-bit ADC, ultra-low-power design

- Temperature Sensor: NTC 10kΩ thermistor (Vishay NTCALUG02A103F, 3950K B-value)

- Reference Circuit: Precision voltage divider (1% resistors, 100nF bypass)

- Anti-Aliasing Filter: 2nd-order Sallen-Key lowpass (~2kHz cutoff)

- Data Path: Sigma-delta conversion, on-chip oversampling

Fault Detection Strategy (Progressive):

- Layer 1 - Hardware: Open/short circuit detection via voltage monitoring

  - Open circuit: ADC reads > 3.2V (divider disconnected)

  - Short circuit: ADC reads < 0.1V

- Layer 2 - Digital Filtering:

  - CRC-16 on each measurement

  - Timeout flag if no update >120ms

- Layer 3 - Intelligent Detection:

  - Kalman filter to track expected trajectory

  - Saturation detection (ADC clipping)

  - Rate-of-change checking (flag if ΔT > 5°C per 100ms as anomaly)

Abnormal Measurement Detection:

- Dual-threshold approach:

  - Primary: ±2.5σ from exponential moving average

  - Secondary: Rate-of-change > 10°C/sec (physical impossibility indoors)

  - Quaternary: Stuck-value detection (same reading >5 consecutive samples)

- Adaptive thresholds based on calibration data

Main Risks & Mitigation:

- Thermistor Self-Heating: Use <1mA excitation current, pulsed ADC sampling

- Nonlinearity Error: Steinhart-Hart 3-coefficient calibration, ±0.5°C accuracy target

- Aging/Calibration Drift: Store calibration constants in EEPROM, periodic recalibration routine

- Temperature-Dependent Reference: Use thermistor itself as reference (ratiometric measurement)

- Sensor Open/Short Worst-Case: Graceful degradation—log error, keep last valid reading, retry

- EMI from Digital Switching: Separate analog/digital grounds, star-point distribution

- Power Supply Instability: Bandgap voltage reference, monitor supply voltage, flag if <2.8V

Comparison

Zero-Shot provides a generic design with standard components and basic fault detection strategies.

One-Shot provides the design to incorporate specific techniques or concepts from the given  humidity sensor example - the output adopts CRC-16 checksum and timeout detection pattern directly from the humidity sensor example and switches component selection to thermistor to match the example's approach.

Three-Shot combines techniques from all three examples to create the design - the LLM selects nRF52840 MCU from the temperature logger example, implements 5-layer fault detection combining pressure (open/short), humidity (CRC/timeout), and temperature (Kalman filter) approaches, and adds adaptive anomaly detection with rate-of-change and stuck-value checking.

Conclusion: Three-shot prompting produces the most production-ready design with best practices from multiple sensor domains applied appropriately to the temperature acquisition context.

2. Justify why you selected these examples (the examples shouldn’t be inserted arbitrarily).

## EXAMPLE 1: Humidity Sensor

Relevance to Temperature Task: LOW

- Different sensor type (measures humidity, not temperature)

- Different sensing domain (moisture vs thermal)

Interface Type: Digital Protocol (I2C/UART)

- Temperature task: Analog signal (ADC sampling)

- Not directly applicable to temperature

What Model Should Learn: Generic digital sensor fault detection patterns that transfer across ANY digital sensor:

- CRC checksum validation

- Timeout detection logic

- UART data logging structure

Why Include It: Teaches fault detection principles before applying them to analog temperature sensors.

## EXAMPLE 2: Pressure Sensor

Relevance to Temperature Task: LOW

- Different sensor type (measures pressure, not temperature)

- Different sensing domain (mechanical vs thermal)

Interface Type: Analog with ADC (12-bit analog-to-digital)

- Temperature task: Also requires 12-bit ADC ✓

- Interface specification MATCHES task

What Model Should Learn: Analog sensor-specific fault detection that applies to any analog sensor:

- ADC saturation detection (when output clips at max/min)

- Open circuit detection (ADC reads max voltage)

- Short circuit detection (ADC reads min voltage)

- Analog filtering principles

Why Include It: Teaches ADC expertise needed for analog temperature sensors without temperature-specific noise.

## EXAMPLE 3: Temperature Logger

Relevance to Temperature Task: HIGH

- Exact sensor type (measures temperature)

- Same sensing domain (thermal)

- Same signal type (analog temperature sensor)

Interface Type: Analog with ADC + Thermistor Circuit

- Temperature task: Requires thermistor + ADC handling ✓

- Directly applicable

What Model Should Learn: Temperature-specific circuit design and compensation:

- Thermistor nonlinearity correction (Steinhart-Hart calibration)

- Self-heating mitigation (thermistor generates heat when current flows)

- Ratiometric measurement (use reference resistor)

- Power optimization for temperature applications

Why Include It: Teaches domain-specific solution for THIS exact problem.

Extra credit (10 points): Conduct a small example-selection experiment comparing different few-shot example sets or orderings and determine whether example choice significantly affects performance.

Prompt D (IRRELEVANT EXAMPLES):

EXAMPLE 1 - LED Brightness Control System:
 Microcontroller: Arduino Uno → PWM LED driver → optical feedback sensor
 Fault detection: LED current limit checks, brightness thresholds exceeded
 Output: Adjusts brightness based on commands, prevents overheating

EXAMPLE 2 - Motor Speed Control System:
 Microcontroller: STM32F4 → DC motor with PWM driver → tachometer feedback
 Fault detection: Speed range limits, stall detection, over-current checks
 Output: Maintains target RPM, logs motor anomalies

EXAMPLE 3 - Servo Position Control System:
 Microcontroller: ATmega328P → Servo actuator → angle encoder feedback
 Fault detection: Position limit checks, calibration verification, servo jam detection
 Output: Moves to target angle, reports position anomalies

Now design a temperature data-acquisition system with these specs:

- 10 Hz sampling rate

- 12-bit ADC, 3.3V supply

- Detect abnormal temperature readings

- Include: architecture, components, fault detection, risks

Output:

## Temperature Data-Acquisition System (From Irrelevant Examples)

Architecture Overview:

- Microcontroller: Arduino Uno (following example pattern, though underpowered)

- Sensor: Generic analog thermistor with voltage divider

- ADC: Built-in 10-bit ADC (mismatches 12-bit requirement)

- Feedback: Simple threshold-based LED status indicator

Key Components:

- Arduino Uno microcontroller

- Thermistor (unspecified, generic)

- Voltage divider resistors

- LED indicator for threshold breaches

- Serial output to monitor

Fault Detection Strategy:

- Position limit checks (adapted from servo example - not applicable)

- Brightness threshold logic (adapted from LED example - not applicable)

- Stall detection concept (misapplied from motor control)

- Over-current checks on wrong domain

Abnormal Measurement Detection:

- Temperature range checks (10°C to 50°C hard limits)

- LED blinks when out of range

- Serial log reports when thresholds exceeded

Main Risks:

- Arduino Uno underpowered for industrial applications

- 10-bit ADC insufficient (task requires 12-bit)

- Feedback mechanisms designed for actuation, not sensing

- No CRC, timeout detection, or sophisticated fault checking

- LED-based feedback unsuitable for sensor monitoring

Issues: Missing ADC specification match, wrong fault detection patterns transferred from motor/servo control.

Prompt E (ONLY ANALOG EXAMPLES):

EXAMPLE 1 - Pressure Sensor (Analog):
 Microcontroller: STM32L476 → 4-20mA transducer → 12-bit ADC
 Fault detection: Saturation detection (ADC clipping), open-circuit check (signal loss)
 Output: Digital pressure reading with error flags

EXAMPLE 2 - Voltage Monitor System:
 Microcontroller: STM32L4 → Precision voltage divider → 12-bit ADC
 Fault detection: Over/under voltage thresholds, ADC range checks, reference voltage drift
 Output: Logs supply voltage anomalies, triggers alerts on deviation

EXAMPLE 3 - Current Sensor (Analog):
 Microcontroller: nRF52840 → Hall effect current sensor → 12-bit ADC
 Fault detection: Current saturation, open sensor detection, zero-current baseline drift
 Output: Analog current measurement with diagnostic flags

Now design a temperature data-acquisition system with these specs:

- 10 Hz sampling rate

- 12-bit ADC, 3.3V supply

- Detect abnormal temperature readings

- Include: architecture, components, fault detection, risks

Output:

## Temperature Data-Acquisition System (From Analog Examples)

Architecture Overview:

- Microcontroller: STM32L476 (proven 12-bit ADC implementation)

- Sensor: NTC Thermistor with analog output

- ADC Interface: 12-bit ADC with sigma-delta capability

- Output: Digital temperature reading with error flags

Key Components:

- MCU: STM32L476 with integrated 12-bit ADC, 3.3V operation

- Sensor: NTC thermistor (10kΩ nominal)

- Signal Conditioning: Voltage divider + RC low-pass filter (cutoff 2kHz)

- Power Supply: 3.3V regulator with bypass capacitors

- Interface: UART or SPI for data logging

Fault Detection Strategy:
 Following analog sensor patterns from examples:

- Saturation Detection: Monitor ADC for max/min values indicating clipping

- Open Circuit Check: Voltage divider output > 3.2V = open thermistor

- Sensor Baseline Drift: Track zero-current reference on startup

- ADC Range Checks: Validate readings within valid sensor range

- Reference Voltage Monitoring: Flag if internal reference drifts

Abnormal Measurement Detection:

- Over/under temperature thresholds (outside -10°C to +50°C)

- Rate-of-change limits (flag if ΔT > 3°C per sample)

- ADC saturation detection (clipping indicates fault)

- Rolling average with ±2.5% deviation band

Main Risks & Mitigation:

- Sensor Nonlinearity: Mitigate with lookup table (not specialized)

- ADC Quantization: Use oversampling to 14-bit effective resolution

- Reference Drift: Monitor internal bandgap reference voltage

- EMI: Separate analog/digital grounds, shield sensor lines

- Power Supply Noise: Multi-stage filtering with ferrite beads

Strengths: Proper 12-bit ADC usage, solid analog fault detection patterns.
 Weakness: Generic analog design, lacks temperature-specific optimizations (no Steinhart-Hart, no thermistor compensation).

Prompt F (ONLY TEMPERATURE EXAMPLES):

EXAMPLE 1 - RTD Temperature Logger:
 Microcontroller: STM32L476 → PT100 RTD with excitation circuit → 12-bit ADC
 Fault detection: RTD lead resistance check, open-circuit detection, resistance ratio validation
 Calibration: Two-point calibration method (ice water, boiling water)

EXAMPLE 2 - Thermistor-Based Temperature System:
 Microcontroller: nRF52840 → NTC 10kΩ thermistor → sigma-delta ADC
 Fault detection: Thermistor short/open circuit detection, self-heating current monitoring
 Calibration: Steinhart-Hart 3-coefficient compensation, nonlinearity correction

EXAMPLE 3 - Digital Temperature Data Logger:
 Microcontroller: ARM Cortex-M4 → DS18B20 digital sensor → CRC validation
 Fault detection: CRC checksum validation, reading timeout detection, sensor presence verification
 Feature: Stores calibration constants in EEPROM, graceful degradation on sensor error

Now design a temperature data-acquisition system with these specs:

- 10 Hz sampling rate

- 12-bit ADC, 3.3V supply

- Detect abnormal temperature readings

- Include: architecture, components, fault detection, risks

Output:

## Temperature Data-Acquisition System (From Temperature Examples)

Architecture Overview:

- Microcontroller: nRF52840 (ultra-low-power, proven for temperature logging)

- Sensor: NTC 10kΩ thermistor with precision voltage divider

- ADC: Sigma-delta 14-bit ADC (converted to 12-bit)

- Calibration: Steinhart-Hart stored in EEPROM

- Output: CRC-validated data with graceful error handling

Key Components:

- MCU: nRF52840 with sigma-delta ADC, ultra-low-power design

- Sensor: NTC thermistor (Vishay NTCALUG02A103F, B=3950K)

- Precision Voltage Divider: 1% tolerance resistors, temperature-compensated

- Reference Circuit: Bandgap reference + ratiometric measurement

- Data Storage: EEPROM for calibration constants (Steinhart-Hart coefficients)

- Communication: SPI/UART with CRC-16 validation

Fault Detection Strategy (Multi-Layer):

Layer 1 - Hardware Detection:

- Thermistor open circuit: Voltage divider output > 3.2V

- Thermistor short circuit: ADC reads < 50mV

- Excitation current monitoring: Detects self-heating anomalies

Layer 2 - Digital Validation:

- CRC-16 checksum on each measurement

- Reading timeout: Flag if no update >120ms

- ADC saturation detection (clipping at boundaries)

Layer 3 - Intelligent Analysis:

- Kalman filter to track expected temperature trajectory

- Rate-of-change checking: Flag if ΔT > 5°C per 100ms

- Stuck-value detection: Same reading >5 consecutive samples

Abnormal Measurement Detection:

- Adaptive thresholds using exponential moving average

- ±2.5σ from baseline (calculated per session)

- Secondary rate-of-change limits (physically impossible rates)

- Tertiary stuck-value detection

Calibration Strategy:

- Steinhart-Hart 3-coefficient method (highest accuracy)

- Ratiometric measurement approach (temperature compensation built-in)

- Two-point calibration points stored in EEPROM

- Graceful degradation: Last valid reading kept on error

Main Risks & Mitigation:

- Thermistor Aging: Periodic recalibration routine, drift tracking

- Self-Heating: Use <1mA excitation, pulsed sampling

- Lead Resistance: Ratiometric approach auto-compensates

- Temperature Dependency: Thermistor itself as reference (self-compensating)

- MCU Reset: Store calibration in EEPROM, auto-restore on startup

- Power Supply Variation: Ratiometric measurement ratio-independent

### FINDINGS

- Irrelevant Examples Catastrophically Hurt Performance

  - Irrelevant design chose Arduino Uno (underpowered)

  - Only 10-bit ADC (task requires 12-bit)

  - Applied LED/motor feedback patterns to temperature sensing

- Analog Examples Improve but Don't Specialize

  - Correct MCU (STM32L476) and 12-bit ADC

  - Solid analog fault detection patterns

  - BUT: Missing thermistor-specific knowledge (no Steinhart-Hart)

- Temperature Examples Enable Excellence

  - Optimal MCU (nRF52840 ultra-low-power)

  - All temperature optimizations present (Steinhart-Hart, EEPROM, Kalman filter)

  - CRC validation, graceful degradation

Therefore, example relevance SIGNIFICANTLY affects design quality: